| LHM |  |
| --- | --- |
|  | 03.09.2001 |

APPLICATION

Castrol LHM is specifically designed for use in powered hydraulic systems, which generally incorporate a pump which drives such components as brake actuation, suspension levelling and power steering.

DESCRIPTION

Castrol LHM is a mineral based fluid specially for Citroen hydraulic systems with green reservoirs.

FEATURES AND BENEFITS

Castrol LHM has a high viscosity index (VI) which means that the product changes

less with temperature than similar products

Castrol LHM is very shear stable which means that the unique viscometric

properties of the product are maintained throughout the service life

Castrol LHM is fully compatible with all elastomers used in existing automotive

powered hydraulic systems recommending this fluid type. It contains corrosion

protection additives in addition.

PERFORMANCE

Citroen B71 2710

TYPICAL CHARACTERISTICS

Relative Density @ 20º C	0.835

Viscosity @  40º C mm²/s	20

Viscosity @ 100º C mm²/s	6.0

Viscosity Index	300

Flash Point ºC	110

Pour Point  ºC	 -50

Colour	Clear green liquid