| Response Super DOT 4 |  |
| --- | --- |
|  | 03.09.2001 |

DESCRIPTION

Castrol Response Super DOT 4 is a high boiling synthetic brake fluid which far exceeds the requirements of the SAE J1703, FMVSS 116 DOT 3 and 4, ISO 4925 and JIS K2233 specifications. Castrol Response Super DOT 4  Brake Fluid is designed for use in all brake systems particularly those which are exposed to extreme conditions. This product has been formulated from mixed polyalkylene glycol ethers and borate ester together with other high performance additives and inhibitors which give ultimate system protection against the effects of corrosion and high temperature vapour lock.

APPLICATION

Castrol Response Super DOT 4  has been specifically designed to be a high temperature alternative to DOT 5.1 fluids. Its superior characteristics give added benefit in situations where extreme conditions are encountered and ultimate braking response required.

Castrol Response Super DOT 4  is based on glycol ether and borate ester technology and as such has a high vapour lock point (measurement relating to high temperature performance under braking conditions). Because of the nature of this formulation, the vapour lock point can be sustained at a higher level than  conventional glycol ether based fluids during the service life of the product .

Castrol Response Super DOT 4  should not be used in braking systems for which a mineral oil based fluid is recommended (for example, some Citroen systems, for which Castrol LHM is suitable and Rolls Royce systems, for which Castrol HSMO Plus is approved).

As with all brake fluids which contain glycol ethers, care should be taken to avoid spilling this product on paintwork, as it may have a damaging effect. In case of spillage, rinse the affected area with water immediately: Do not wipe.

It is fully compatible with other fluids meeting FMVSS 116 DOT 3, DOT 4 and DOT 5.1 - however, in order to maintain the superior performance characteristics of Castrol Response Super DOT 4 avoid mixing with other brake fluid products.

| All reasonable care has been taken to ensure that the information contained in this publication is accurate as at the date of printing. It should be noted however that the information above may be affected by changes occurring subsequent to the date of printing in the blend formulation or methods of application of any of the products referred to or in the requirements of any specification approval relating to any such products. | 1 of 2 |
| --- | --- |

| Response Super DOT 4 |  |
| --- | --- |

All conventional brake fluids deteriorate during use.

It is strongly recommended that Castrol Response Super DOT 4 should be changed according to the vehicle manufacturers advice. In the absence of such advice, a 2 year change period is recommended.

PERFORMANCE

SAE J1703 (Current)

FORD ESD M6C 57A

ISO 4925

FMVSS 116 DOT 3 & DOT 4

JIS K 2233

TYPICAL CHARACTERISTICS

Dry Boiling Point ºC	280

Wet Boiling Point ºC	186

Viscosity at -40ºC, mm²/s	1200

pH	7.2

Density at 20ºC (g/ml)	1.07